Please put everything in into Latex IEEE format, Follow the following structure for sections. Label all the math as equations (1) (2) and so on. Also write dummy captions for each figure I’ll write the captions by myself.

Introduction

Now a days 3d printers are commonly being used in different applications. There are different types of 3d printers based on their mechanism. Simply supported beam with moving bed is one of such configurations which works on independent joint control mechanism. In independent joint control the control of each axis of the manipulator is controlled as a singleinput/ single-output (SISO) system. Any coupling effects due to the motion of the other links is treated as a disturbance. In the above described 3d printer, it consists of the following mechanism.

- Printer bed: A flat surface on which the objects get printed.

- Extruder: The part that heats up and dispenses material used for printing the 3D object.

- Motor X : Moves the printer bed in the x direction. The motor drives the printer bed through a gearing and a ballscrew/ belt mechanism. The mechanism and printer bed acts as a load for this motor.

- Motor Y: Moves the extruder in the y direction. The motor drives the extruder through a gearing and a ball-screw/belt mechanism. The mechanism and extruder acts as a load for this motor.

- Motor Z: Moves the printer structures in the z direction. The motor moves the structure through a gearing and a ball-screw/belt mechanism. The structure and mechanism act as a load for this motor.

Mathematical Modeling

In order to derive the mathematical model of the motor one must know the dynamics of motor. Block diagram of a single axis DC Motor drive system is shown in Figure:fig1. As shown, motor consists of three parts, electrical part, motor side mechanical part and load side mechanical part.

Electrical part

Page 1 of Derivation till eq(1)

Mechanical part

Page 1 of Derivation till eq(2)

Load side mechanical part

Page 2 of Derivation till eq(3)

Solving equations using MATLAB

Page 2 from eq(2) – eq(3) till end

Also add he following chunk of codes in MATLAB coding style (code should stand out as code differ from normal text)

clc

clear all

%--Solving for X_L/Em by setting D=0

syms s Em b D1 D2 Jmot Jlod Kb Kt Kr Ks La Ra mL N1 N2 Ia XL r

D=0;

eq2= (((Jmot/Kr)*(r)*sˆ2+(D1/Kr)*(r)*s))/XL == Kt*Ia

eq3= (((Jlod/Kr)-Ks*mL)*sˆ2+((D2/Kr)*(Ks*b)*s))/XL == Ks*D

temp=eq2-eq3

Ia=solve(temp,Ia)

eq1= (La*s+Ra)*Ia == Em - (Kb/Kr)*(r)*s/XL

vars=[XL Em]

[E_m, X_L]=solve(eq1,vars)

out=X_L/E_m

out=simplify(out,1000)

pretty(out)

%%

%--Solving for X_L/Em by setting Em=0

syms s D b D1 D2 Jmot Jlod Kb Kt Kr Ks La Ra mL N1 N2 Ia XL r

Em=0;

eq1= (La*s+Ra)*Ia == Em - (Kb/Kr)*(r)*s/XL

Ia=solve(eq1,Ia)

eq2= (((Jmot/Kr)*(r)*sˆ2+(D1/Kr)*(r)*s))/XL == Kt*Ia

eq3= (((Jlod/Kr)-Ks*mL)*sˆ2+((D2/Kr)*(Ks*b)*s))/XL == Ks*D

temp=eq2-eq3

vars=[XL D]

[Dis, X_L]=solve(temp,vars)

out2=X_L/Dis

out2=simplify(out2,1000);

pretty(out2)

Feedback control systems

Designing of individual feedback back control system is done using root locus design tool. The same configuration shown in Figure:model is used for X, Y and Z motors. All the controllers are tuned to make sure all the motors have similar transient response which has a settling time of less than 1 second and an overshoot of less than 5%. The design process of each motor controller is discussed below.

Y motor controller design

The open loop poles of the system are:

- 0

- -2.4472

- -0.0530

The first pole at 0 making the system marginally stable and is the most dominant pole. The root locus of the system is shown in Figure:rootlocus_X. Thus, when the value of gain increases, it move the poles to the unstable region. The aim is to reduce the settling time and increase the damping. Technically shifting the poles to left is also desirable i.e. moving the marginally stable pole to the stable region. Lead compensator via root locus design has been used because it increases relative stability by increasing phase margin. For a given system gain, lead compensator increase steady state error. To decrease steady state error, large compensator gain must be used. The additional effect of this it decreases the step response settling time, i.e. increasing the system damping. Moreover, lead compensator also shifts the poles towards left half of the Real-Imaginary axis. First order Lead compensator doesn’t meet the required specifications because the system has very less gain. Therefore, 2nd order Lead compensator is used i.e cascading two lead compensator. The tuned parameters through root locus design of the controller is shown in Figure:comp_X. The resulting root locus after tuning the compensator is shown in Figure:rootlocus_X2. In which pole zero cancellation removed the effect of dominant poles near to the origin resulting in large gain margin that helped in achieving the desired response. The step response of closed loop Y motor system is shown as Figure:step_X.

Y motor controller design

The open loop poles of the X motor plant are:

- 0

- -2.3791

- -0.1210

The first pole at 0 making the system marginally stable and is the most dominant pole. The root locus of the system is shown in Figure:rootlocus_Y. This case is also similar to Y motor. Thus, applying 2nd order Lead compensator is used i.e cascading two lead compensator. The tuned parameters through root locus design of the controller is shown in Figure:comp_Y. The resulting root locus after tuning the compensator is shown in Figure:rootlocus_Y2. In which pole zero cancellation removed the effect of dominant poles near to the origin resulting in large gain margin that helped in achieving the desired response. The step response of closed loop X motor system is shown as Figure:step_Y.

Z motor controller design

The open loop poles of the Z motor plant are:

- 0

- -2.0170

- -0.4830

The first pole at 0 making the system marginally stable and is the most dominant pole. The root locus of the system is shown in Figure:rootlocus_Z. This case is also similar to X and Y motor. Thus, applying 2nd order Lead compensator is used i.e cascading two lead compensator. The tuned parameters through root locus design of the controller is shown in Figure:comp_Z. The resulting root locus after tuning the compensator is shown in Figure:rootlocus_Z2. In which pole zero cancellation removed the effect of dominant poles near to the origin resulting in large gain margin that helped in achieving the desired response. The step response of closed loop Z motor system is shown as Figure:step_Z.

Square wave response

Applying continuous step input i.e. square waves using pulse generator having period of 3 seconds and pulse width of 50%. The output response to the square wave is shown in Figure:square.

Effect of constant disturbance

Applying constant disturbance of 5N does not effect the output response of the system as shown in Figure:dis_out, which means the feedback controller of every motor can compensate the effect of the applied disturbance.

Moreover, a comparative plot of step response with and without disturbance is shown in Figure:d_with_No. But a constant force of more than 100N results in slight deviation from the reference input as shown in Figure:Err_plot_100N which is zoomed to 300%.

3d printing without disturbance

The following MATLAB code has been used to read data from pseudo .stl file and saving as .mat file for further use.

dataset=xlsread(’pseudo stl file data.xlsx’,’CakeRaw’);% read excel file

x=dataset(1:155,2)%specify rows and columns to read from the file

y=dataset(1:155,3)

z=dataset(1:155,4)

dispense=dataset(:,5)

stop=dataset(:,6)

incr=dataset(1:155,7)

save (’read_cakeRaw.mat’, ’x’,’y’,’z’,’incr’) %saving variables in .mat file for further use

Similar code as above has been used to read all the spread sheets from excel file and save it as .mat file. Following code has been used to simulate the model in Simulink by giving input from MATLAB script and taking output back into MATLAB script and plot it. Error tolerance hasn’t been catered because the error was already below 0.05 and unsynchronization of Simulink time with MATLAB function code loops. Moreover, the input to the simulink model is exactly after 1 second as incr in .stl is treated as time axis, which is greater than the settling time, therefore, the output printed object is doesn’t show any large variation.

load read_cylinder.mat %load .mat file

simin_x=[incr,x]; %passing data to Workspace as Timeseries, where incr is time and x are coordinates for X Mator

simin_y=[incr,y];

simin_z=[incr,z];

sim(’temp2’,length(incr)); %running simulation temp2.mdl with simulation time equal to the time scale of .stl file

plot3(simout_x,simout_y,simout_z) %plot 3D on data obtained from ToWorkSpace block of simulink

title(’Cylinder with constant D(s)’)

xlabel(’x-axis (meters)’)

ylabel(’y-axis (meters)’)

zlabel(’z-axis (meters)’)

grid on

figure

plot(err_x) %plot of error from E(s) in feedback loop of Motor X

title(’Motor X Error Plot with D(s)’)

ylabel(’Displacement (m)’)

grid on

figure

plot (err_y) %plot of error from E(s) in feedback loop of Motor Y

title(’Motor Y Error Plot with D(s)’)

ylabel(’Displacement (m)’)

grid on

figure

plot (err_z) %plot of error from E(s) in feedback loop of Motor Z

ylabel(’Displacement (m)’)

title(’Motor Z Error Plot with D(s)’)

grid on

3d printing of Rectangle

The 3d plot of Rectangle shown in Figure:rectangle_print without any disturbance. The edges are slightly circular when the figure is zoomed because of error tolerance. The error plots of X, Y and Z motors in case of rectangle in shown Figure:rectangle_errx, Figure:rectangle_erry and Figure:rectangle_errz respectively.

3d printing of Cylinder

The 3d plot of Cylinder shown in Figure:cylinder_print without any disturbance. No error can be visualized in cylinder as it has no sharp edges for further eleboration the error plots of X, Y and Z motors are shown Figure:cylinder_errx, Figure:cylinder_erry and Figure:cylinder_errz respectively.

3d printing of Cube

The 3d plot of Cube shown in Figure:cube_print without any disturbance. The edges of Cube is not sharp enough when the figure is zoomed because of error tolerance. The error plots of X, Y and Z motors in case of Cube in shown Figure:cube_errx, Figure:cube_erry and Figure:cube_errz respectively.

3d printing of Pyramid

The 3d plot of Pyramid shown in Figure:pyramid_print without any disturbance. The edges of Pyramid is not sharp enough when the figure is zoomed because of error tolerance. The error plots of X, Y and Z motors in case of Pyramid in shown Figure:pyramid_errx, Figure:pyramid_erry and Figure:pyramid_errz respectively.

3d printing with disturbance of 500N

3d printing of Rectangle with D(s)

The 3d plot of Rectangle shown in Figure:rectangle_print_D with a constant disturbance of 500N applied on each X, Y and Z motor. The edges are slightly circular when the figure is zoomed because of error tolerance. The error plots of X, Y and Z motors in case of rectangle in shown Figure:rectangle_errx_D, Figure:rectangle_erry_D and Figure:rectangle_errz_D respectively. Although error increases with disturbance but was quiet small to produce any visual change in the printed object.

3d printing of Cylinder with D(s)

The 3d plot of Cylinder shown in Figure:cylinder_print_D without any disturbance. The change in layers of cylinder is not sharp enough when the figure is zoomed because of error tolerance. The error plots of X, Y and Z motors in case of cylinder in shown Figure:cylinder_errx_D, Figure:cylinder_erry_D and Figure:cylinder_errz_D respectively. Although error increases with disturbance but was quiet small to produce any visual change in the printed object.

3d printing of Cube with D(s)

The 3d plot of Cube shown in Figure:cube_print_D without any disturbance. The edges of Cube is not sharp enough when the figure is zoomed because of error tolerance. The error plots of X, Y and Z motors in case of Cube in shown Figure:cube_errx_D, Figure:cube_erry_D and Figure:cube_errz_D respectively. Although error increases with disturbance but was quiet small to produce any visual change in the printed object.

3d printing of Pyramid with D(s)

The 3d plot of Pyramid shown in Figure:pyramid_print_D without any disturbance. The edges of Pyramid is not sharp enough when the figure is zoomed because of error tolerance. The error plots of X, Y and Z motors in case of Pyramid in shown Figure:pyramid_errx_D, Figure:pyramid_erry_D and Figure:pyramid_errz_D respectively. Although error increases with disturbance but was quiet small to produce any visual change in the printed object.

Comparative error plots

Comparative plots of error of motor X, Y and Z while printing Cube are shown in Figure:dis_X_final, Figure:dis_Y_final, Figure:dis_Z_final. When the disturbance of 500N is applied , the error increases but unable to vary the settling time because of the designed compensator, therefore, slight variation doesn’t effect the final printed object.

Conclusion

The mathematical modeling of transfer function of X, Y and Z motors along with its gear and load has been performed. Then a Lead controller has been designed to control each X, Y and Z feedback loop individually. Simulation results proved that the controller can compensate a disturbance of upto 500N which is a quiet large value in terms of 3d printer. Moreover, 3d objects has been printed using designed 3d printer model by giving input coordinates from .stl file.

List of symbols

References